SAFE WORK METHOD STATEMENT – WORKING AT HEIGHT

| RISK MATRIX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - CONSEQUENCE - evaluate the consequences of a risk occurring according to the ratings in the Consequence table i.e. low, moderate, high, or extreme. |  |  |  |  |  |  |  |  | - LIKELIHOOD - evaluate the likelihood of an incident occurring according to the ratings in the left-hand column i.e. rare, unlikely, possible, likely, or almost certain. |  |  |  |  |  |
| DESCRIPTOR |  | LEVEL | DEFINITION |  |  |  |  | DESCRIPTOR |  | LEVEL |  |  | DEFINITION |
| Insignificant |  | 1 | No injury |  |  |  |  | Rare |  | 1 |  |  | May occur somewhere, sometime (“once in a lifetime / once in a hundred years”) |
| Minor |  | 2 | Injury/ ill health requiring first aid |  |  |  |  | Unlikely |  | 2 |  |  | May occur somewhere within the workplace over an extended period of time |
| Moderate |  | 3 | Injury/ill health requiring medical attention |  |  |  |  | Possible |  | 3 |  |  | May occur several times across the workplace or a region over a period of time |
| Major |  | 4 | Injury/ill health requiring hospital admission |  |  |  |  | Likely |  | 4 |  |  | May be anticipated multiple times over a period of time May occur once every few repetitions of the activity or event |
| Severe |  | 5 | Fatality |  |  |  |  | Almost certain |  | 5 |  |  | Prone to occur regularly It is anticipated for each repetition of the activity of event |
| - RISK MATRIX - Using the Matrix below, calculate the level of risk by finding the intersection between the likelihood and the consequence |  |  |  |  |  |  |  |  | - RISK LEVEL / RATING AND ACTIONS |  |  |  |  |  |
| LIKELIHOOD | CONSEQUENCE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Insignificant |  |  | Minor | Moderate | Major | Severe |  | DISCRIPTOR |  |  | DEFINITION |  |  |
| Almost Certain | 1E Moderate |  |  | 2E High | 3E Extreme | 4E Extreme | 5E Extreme |  | Extreme>> |  |  | Notify workplace manager and/or management OHS nominee immediately. Corrective actions should be taken immediately. Cease associated activity. |  |  |
| Likely | 1D Moderate |  |  | 2D Moderate | 3D High | 4D Extreme | 5D Extreme |  | High>> |  |  | Notify workplace manager and/or management OHS nominee immediately. Corrective actions should be taken within 48 hours of notification. |  |  |
| Possible | 1C Low |  |  | 2C Moderate | 3C Moderate | 4C High | 5C Extreme |  | Moderate>> |  |  | Notify nominated worker, HSR / HSC. Nominated worker, OHS representative / HSC is to follow up that corrective action is taken within 7 days. |  |  |
| Unlikely | 1B Low |  |  | 2B Low | 3B Moderate | 4B Moderate | 5B High |  | Low>> |  |  | Notify nominated worker, HSR / HSC. Nominated worker, HSR / HSC is to follow up that corrective action is taken within a reasonable time. |  |  |
| Rare | 1A Low |  |  | 1B Low | 1C Low | 1D Moderate | 1E Moderate |  |  |  |  |  |  |  |

| HIERARCHY OF CONTROLS |  |
| --- | --- |
| Develop Controls in accordance with the hierarchy below (also see diagram on the right): - Elimination – Removing the hazard from the workplace e.g. removing damaged equipment and removing unwanted chemicals. - Substitution – Substituting or replacing the hazard with a less hazardous one e.g. using less hazardous chemicals. - Engineering Control – Engineering out the hazard e.g. using a trolley to move objects. - Administrative Control – Introducing work practices that reduce the risk e.g. job rotation and safe work procedures. - Personal Protective Equipment – This is the last control method that should be used e.g. gloves, masks. Staff must be trained to use PPE correctly. |  |

| Job description | Workplace: Warehousing Department | Name of person conducting risk assessment | Date: |
| --- | --- | --- | --- |
|  |  | Name of other workers | Signature |
| Risk Assessment Purpose The purpose of a risk assessment is to identify the risks associated with a task, activity, or process, and put appropriate controls in place to eliminate or reduce those risks. Completing a risk assessment helps ensure that hazards, risks and controls are documented and communicated to those who will be involved in the task, activity, or process. NB: Residual risk level estimate for each item is dependent on implementation of other related controls listed against other related hazards. |  |  |  |

| Possible Hazards | Identified / Yes | Control to be used | Risk level after control |
| --- | --- | --- | --- |
| Personal protective equipment |  |  |  |
| Surface stability |  |  |  |
| - Is the work being undertaken on a fragile surface? |  | - Work from a solid construction e.g., reinforcing the structure before work commences (if so use other to explain control) - Use of EWP - Installing edge protection barricades - Use of fall protection harness - Other _____________________________________________________________________________________________________________________________________________________________________________________________________________ |  |
| - Are you working on an angled surface e.g. roof pitch greater than 15 degrees? |  |  |  |
| - Is the surface slippery e.g., due to wet conditions? |  |  |  |
| Equipment |  |  |  |
| - Equipment being used for work at height has NOT been inspected, tested or certified. |  | - Equipment has been replaced with serviced & certified equipment - Ensure tools and equipment have been secured so they cannot fall from height - All terrain EWPs are being used and level alarms are in correct working order - Operators are licensed to use the equipment - Other _____________________________________________________________________________________________________________________________________________________________________________________________________________ |  |
| - Are you operating Elevated Work Platforms on uneven ground or significant slopes? |  |  |  |
| - Are restricted places to rest tools when working at height |  |  |  |
| - Is there a chance that plant and equipment could fall on people below? |  |  |  |
| Harnesses and Ladders |  |  |  |
| - Harnesses are NOT being used in accordance with the controls list (Confirm you are using all the controls listed in the controls column by checking the box) |  | - The following controls are in place for harness use as required: - Each anchor point component complies with required standards - All anchor points have been tested and approved by a competent person before use Note: a visual inspection may not reveal the structural integrity of the anchor point e.g. subsurface fractures etc. - Detailed installation specifications are being followed (by competent persons) for the type of anchorage and location. Type of anchor. Consider: - Type of anchor required - Purpose of anchor (free fall, limited free fall, horizontal lifelines, restraint techniques etc.) - Type of work being undertaken - Fastening devices (ensure all bolts, nuts, and washers meet required maximum breaking strength for shear (lateral) and axial (tension) directions - Welds must be subject to non-destructive inspection and testing Placement of anchor. Consider: - Manufacturer’s instructions for installation of anchorages are being followed - Adequate fall distance between operator and lower-level obstructions (machinery, open windows/doors, panels, tree limbs etc.) - Position of workers - Correct position to avoid pendulum effect - Type of likely fall and angle of incidence - Rebound forces - Each anchor point is located so that a lanyard of the system can be attached to it before the person moves into a position where they could fall - You will seek permission from a qualified person if changes to installation design plans change - Installation area have been visually inspected to ensure structure is exactly as described in installation specs (materials and condition) - Anchorages and fasteners are visually inspected - You have ensured that fastening materials will make contact with all surfaces as required - You have checked the anchor is firmly installed as per instructions - You have ensured there are no missing or surplus fasteners - You have not used excessive force when installing - You have ensured that for anchorages that require a sling, the sling is sufficient length for an angle no greater than 120 degrees between the legs (unless specifically designed for higher loads) - Once completed, you have test-loaded strength of anchor as per design specifications - You have re-inspected anchorage and fasteners after testing - A competent person has certified that anchor point meets required standards and safe for use - Anchor points have NOT been fastened to the following unless specifically modified to do so: - Conduit / cable trays - Piping / support racks / Vent stacks - Scaffolding / supports / Suspended platforms - Guards rails / hand rails - You have ensured maximum distance free fall before arrest does not exceed 2m - You have ensured sufficient distance between work surface and any surface below to enable shock absorber to fully deploy Inspections - Static lines and harness are within test 12 month test period and you have visually inspected it before use |  |
| - Ladders are NOT being used in accordance with the controls list (Confirm you are using all the controls listed in the controls column by checking the box) |  | - The following control are in place for ladder use: - Ladders are inspected and erected as per manufacturers’ instructions - Ladder extends at least 1 m above the place of landing of the highest rung to be reached by the feet of any person working on ladder - Ladders are set up on a level area on firm footing with the base located approximately a quarter of the vertical height of the ladder from the wall - Secured against movement - Metallic, wire- reinforced or conductive ladders should be avoided around live electrical equipment - Face ladder when ascending/descending. Ensure 3 points of contact remain on ladder at all times - Load rating of at least 120kg - Correct size and length for job - No damage, clean and dry - Secured to work site (base & top) - Ensure only 1 person working from each ladder |  |
| Unprotected edges |  |  |  |
| - Are you carrying out tasks near unprotected edge at height e.g., closer than 3m? |  | - Holes or voids have been covered that are designed to withstand loads. - We will be using a fall arrest system that is compliant - Other _____________________________________________________________________________________________________________________________________________________________________________________________________________ |  |
| - There is an unprotected edge e.g., near shafts, trenches, pits without guard railing or barricades? |  |  |  |
| Electrical |  |  |  |
| - Is there a chance that you could make contact with overhead electrical equipment? |  | - Electrical equipment has been isolated and isolation has been tested - Area near electrical equipment has been cordoned off as a no-go zone - Other _____________________________________________________________________________________________________________________________________________________________________________________________________________ |  |
| Climatic conditions |  |  |  |
| - Weather conditions pose a risk of fall, for e.g. hail, thunder storms, rain BOM, Strong winds etc |  | - Tasks have been suspended as weather poses a risk - Other _____________________________________________________________________________________________________________________________________________________________________________________________________________ |  |
| Emergency management |  |  |  |
| - Lack of immediate access to medical services |  | - There is an agreed rescue plan in place. - You have access to a first aid kit - Other _____________________________________________________________________________________________________________________________________________________________________________________________________________ |  |
| Other identified hazards |  |  |  |
| Hazards | Controls |  | Risk level |